Summary

Group 15
experiment: Effect of viscosity on heat transfer 

In this experiment, we aimed to characterize convective heat transfer in a helical coil operating under purely laminar flow conditions. The working fluids were water–glycerol mixtures at four concentrations: 10%, 15%, 20%, and 30% (v/v). The system was run at bath temperatures of 50°C and 60°C, and all thermophysical properties were evaluated at the corresponding bulk temperatures of 38.5°C and 43.0°C, respectively.

Using the measured inlet and outlet temperatures, we first calculated the total heat transfer rate from the energy balance:
Q = m·Cp·(T_out − T_in).
From this, we determined the experimental convective heat transfer coefficient h_exp using the LMTD (Log Mean Temperature Difference) method.

Key assumption
A key simplification in our analysis was treating the overall heat transfer coefficient U as approximately equal to h_exp. This assumption is justified because the copper tube wall contributes negligible conductive resistance, and the external heat transfer coefficient h_out is effectively very large due to continuous mechanical stirring in the water bath. As a result, external resistance can be considered negligible.

Validation
To validate our experimental results, we compared them with the correlation proposed by Dravid et al. (1971), which is specifically developed for thermally developing secondary flows in helical coils. The agreement between our data and the correlation was satisfactory.

We also confirmed mathematically that the flow remained in the thermally developing regime. This was done by calculating the thermal entrance length using:
L_e,T = 0.05 · D · Re_D · Pr
which showed that the thermal boundary layer did not fully develop within the 3-meter coil length.

As an additional benchmark, we compared our experimentally obtained h_exp for the helical coil with predictions for a straight pipe under developing flow conditions. The comparison revealed a significant enhancement: the helical coil exhibited approximately 200% higher heat transfer on average. This clearly demonstrates the effect of Dean vortices, which enhance mixing and improve thermal transport.

Revised validation
To further illustrate this, we plotted axial temperature profiles T(x) versus x for both straight and helical configurations. We also constructed a plot of the straight-pipe heat transfer coefficient against the inverse Graetz number Gz⁻¹. The methodology for this plot is important to clarify:

For Gz⁻¹ ≤ 0.05, the flow is thermally developing, so we used the full Hausen correlation to calculate h.
Beyond this point, the flow is effectively fully developed, and we used the limiting Nusselt number Nu = 3.66 to compute h.

Limitations
Despite these successful analyses, there are two important limitations in our methodology that should be acknowledged.

The first limitation arises from the LMTD method itself, which provides only a single average heat transfer coefficient over the entire coil length. Consequently, we were unable to obtain a spatially varying heat transfer coefficient h_exp(x), and therefore could not directly plot it as a function of the Graetz number along the pipe.

The second limitation is more subtle. We could not determine the exact axial position within the coil where the fluid reaches the outlet temperature T_out and effectively stops absorbing heat. Since h_exp is calculated under the assumption that the fluid reaches T_out exactly at the 3.0-meter exit, substituting these values back into the local temperature profile simply returns x = 3.0 m. This creates a circular result that does not provide additional physical insight.

Importantly, these are not shortcomings of the experimental procedure, but rather inherent limitations of the LMTD framework in resolving spatial variations.